# תרגיל 2 – פתרונות | Problem Set 2 – Solutions

## 512.4462 מבוא לתקשורת מחשבים

## Question 1: Spanning Tree Protocol

### Network Topology

| Bridge | Ports and Segments |
| --- | --- |
| B12 | P1→S3, P2→S1 |
| B16 | P1→S5, P2→S1, P3→S4 |
| B24 | P1→S7, P2→S3 |
| B32 | P1→S9, P2→S7, P3→S5, P4→S10 |
| B35 | P1→S8, P2→S6, P3→S4 |
| B55 | P1→S11, P2→S9 |
| B73 | P1→S11, P2→S10, P3→S12 |
| B92 | P1→S4, P2→S2 |

### 1.1 – Normal Spanning Tree

Root Bridge: B12 (lowest ID = 12)

Bridge distances and root ports:

| Bridge | Distance from Root | Root Port | Via Segment |
| --- | --- | --- | --- |
| B12 | 0 | — (root) | — |
| B16 | 1 | P2 | S1 |
| B24 | 1 | P2 | S3 |
| B32 | 2 | P3 | S5 (via B16, ID 16 < B24’s 24 on S7) |
| B35 | 2 | P3 | S4 (via B16) |
| B92 | 2 | P1 | S4 (via B16) |
| B55 | 3 | P2 | S9 (via B32) |
| B73 | 3 | P2 | S10 (via B32) |

Designated ports per segment:

| Segment | Designated Port | Reasoning |
| --- | --- | --- |
| S1 | B12-P2 | Root directly on segment (cost 0) |
| S2 | B92-P2 | Only bridge on segment |
| S3 | B12-P1 | Root directly on segment (cost 0) |
| S4 | B16-P3 | Cost 1 (B16) vs cost 2 (B35, B92) |
| S5 | B16-P1 | Cost 1 (B16) vs cost 2 (B32) |
| S6 | B35-P2 | Only bridge on segment |
| S7 | B24-P1 | Cost 1 (B24) vs cost 2 (B32) |
| S8 | B35-P1 | Only bridge on segment |
| S9 | B32-P1 | Cost 2 (B32) vs cost 3 (B55) |
| S10 | B32-P4 | Cost 2 (B32) vs cost 3 (B73) |
| S11 | B55-P1 | Cost 3, B55 (ID 55 < 73) vs B73 |
| S12 | B73-P3 | Only bridge on segment |

Blocked ports: - B32-P2 (on S7) – not root port, not designated - B73-P1 (on S11) – not root port, not designated

Non-forwarding bridges: None. All bridges have at least one designated port and thus forward frames.

### 1.2 – B32 Thinks It Is B12

B32 sends BPDUs with bridge ID = 12 and cost = 0 (acts as a second root). The network now has two “roots” both claiming ID 12.

BPDU tie-breaking between real B12 and fake B12 (B32): Same root ID, same cost, same bridge ID → compare port IDs. Real B12 uses ports P1, P2; Fake B12 (B32) uses P1, P2, P3, P4.

Results:

| Bridge | Distance | Root Port | Reasoning |
| --- | --- | --- | --- |
| B12 | 0 | — | Real root |
| B32 | 0 | — | Fake root (thinks it’s B12) |
| B16 | 1 | P2 (S1) | Hears (12,0,12,P2) on S1 from real B12 vs (12,0,12,P3) on S5 from B32; P2 < P3 |
| B24 | 1 | P2 (S3) | Hears (12,0,12,P1) on S3 from real B12 vs (12,0,12,P2) on S7 from B32; P1 < P2 |
| B55 | 1 | P2 (S9) | Hears (12,0,12,P1) directly from B32 on S9 |
| B73 | 1 | P2 (S10) | Hears (12,0,12,P4) directly from B32 on S10 |
| B35 | 2 | P3 (S4) | Via B16 on S4 |
| B92 | 2 | P1 (S4) | Via B16 on S4 |

Designated ports:

| Segment | Designated Port | Reasoning |
| --- | --- | --- |
| S1 | B12-P2 | Real root (cost 0, bridge 12, port P2) |
| S2 | B92-P2 | Only bridge |
| S3 | B12-P1 | Real root (cost 0, bridge 12, port P1) |
| S4 | B16-P3 | Cost 1, bridge 16 |
| S5 | B32-P3 | Fake root (cost 0, bridge 12, port P3) beats B16 (cost 1) |
| S6 | B35-P2 | Only bridge |
| S7 | B32-P2 | Fake root (cost 0, bridge 12, port P2) beats B24 (cost 1) |
| S8 | B35-P1 | Only bridge |
| S9 | B32-P1 | Fake root (cost 0, bridge 12, port P1) |
| S10 | B32-P4 | Fake root (cost 0, bridge 12, port P4) |
| S11 | B55-P1 | Cost 1, B55 (ID 55 < 73) |
| S12 | B73-P3 | Only bridge |

Blocked ports: - B16-P1 (on S5) – B32 is designated on S5 - B24-P1 (on S7) – B32 is designated on S7 - B73-P1 (on S11) – B55 is designated on S11

Non-forwarding bridge: B24 – Its only ports are P1 (blocked on S7) and P2 (root port on S3). It has no designated port on any segment, so it does not forward any user frames.

## Question 2: Linear Chain of Bridges

### Setup

Segments 0, 1, …, n connected by bridges 1, 2, …, n (bridge i connects segment i−1 to segment i). Uniform traffic x between every pair of segments.

### 2.1 – Traffic on Bridge i (Linear Chain)

In a linear chain there are no loops, so the spanning tree is the chain itself regardless of which bridge is root. No ports are blocked.

Bridge i carries all traffic that crosses it – i.e., traffic between segments {0, …, i−1} (left side, i segments) and segments {i, …, n} (right side, n+1−i segments).

Total traffic on bridge i:

Traffic(i)=2⋅i⋅(n+1−i)⋅x

(Factor 2 for both directions: left→right and right→left.)

Does it depend on root position? No. Since the topology is a simple line (no loops), the spanning tree equals the full topology regardless of which bridge has the minimum ID. No ports are ever blocked.

### 2.2 – Ring Topology (Bridge 0 Added, Lowest ID)

Bridge 0 connects segment 0 to segment n, forming a ring. Bridge 0 has the smallest ID → it is the root.

Since bridge 0 is root, both its ports (on seg 0 and seg n) are designated. Bridges route toward whichever side reaches bridge 0 faster: - Bridge i at distance min(i, n+1−i) from root - Bridges 1, …, ⌊n/2⌋ route leftward (toward seg 0) - Bridges ⌈n/2⌉+1, …, n route rightward (toward seg n)

If n is even, bridge n/2 has equal distance both ways; tie-break by lower bridge ID on the path (bridge 0 vs bridge n, bridge 0 wins → bridge n/2 routes leftward through seg n/2 − 1… Actually the tie-break is by the BPDU: bridge n/2 hears from bridge n/2−1 (dist n/2−1, going left) vs bridge n/2+1 (dist n/2−1, going right). The tie-break is on the bridge ID of the sender: lower ID wins. For simplicity, assume bridge n/2 routes leftward (toward lower-numbered bridges).

The spanning tree splits into two arms from bridge 0. The blocked port is on the segment where the two arms meet (approximately in the middle).

Traffic on bridge i:

For i ≤ ⌊n/2⌋ (left arm): bridge i separates segments {0,…,i−1} from segments {i,…,⌊n/2⌋} in its subtree direction. However, since the tree is now two arms from a single root:

Bridge i (left arm) carries traffic between: - All segments in the subtree “beyond” bridge i (segments {i, …, ⌊n/2⌋}) - All other segments ({0, …, i−1} ∪ {⌈n/2⌉+1, …, n})

Traffic(i)=2⋅(⌊n/2⌋−i+1)⋅(n+1−(⌊n/2⌋−i+1))⋅x

=2⋅(⌊n/2⌋+1−i)⋅(n−⌊n/2⌋+i)⋅x

By symmetry, for bridge i in the right arm (i > ⌈n/2⌉):

Traffic(i)=2⋅(i−⌈n/2⌉)⋅(n+1−(i−⌈n/2⌉))⋅x

Simplified (for large even n, split at n/2):

- Left arm bridge i (1 ≤ i ≤ n/2): Traffic = 2·(n/2 + 1 − i)·(n/2 + i)·x

- Right arm bridge i (n/2 + 1 ≤ i ≤ n): Traffic = 2·(i − n/2)·(n + 1 − i + n/2)·x

The maximum traffic is on bridges closest to root (i=1 and i=n), approximately n²/2 · x for large n. The middle bridges carry minimal traffic ≈ n·x.

## Question 3: Clos Network Cost Optimization

### Rearrangeable Non-Blocking Clos Architecture

For N×N switch with crossbar components of size c×c: - Stage 1: N/c switches, each c×c → cost per switch: c^(3/2) - Stage 2 (middle): c switches, each (N/c)×(N/c) → cost per switch: (N/c)^(3/2) - Stage 3: N/c switches, each c×c → cost per switch: c^(3/2)

(For rearrangeable non-blocking: number of middle switches m = c is sufficient.)

### Total Cost

Cost(c)=2⋅Nc⋅c3/2+c⋅Nc3/2

=2Nc+N3/2c

### Optimization

Taking derivative and setting to zero:

ddcCost=Nc−N3/22c3/2=0

Nc=N3/22c3/2⟹2c=N

c*=N2

### Minimum Cost (for large N)

Substituting c* = √N/2:

Costmin=2N⋅N1/42+N3/2⋅2N1/4=2N5/42+2⋅N5/4=22⋅N5/4

## Question 4: Switch Scheduling

### 4.1 – External Scenario: Feasible with OQ, Infeasible with CIOQ (no speedup)

Switch: 2×2

| Time Slot | Arrivals |
| --- | --- |
| t = 1 | Pkt A: input 1 → output 1; Pkt B: input 2 → output 1 |
| t = 2 | Pkt C: input 2 → output 2 |

With Output Queuing (OQ): - t=1: Fabric transfers both A and B to output 1’s queue (fabric speed = N × line speed). A departs output 1 at t=1. - t=2: B departs output 1 at t=2. C transfers to output 2 queue, departs at t=2. - Departure schedule: A@1, B@2, C@2. ✓ Feasible.

With CIOQ (no speedup): - t=1: Partial permutation – at most one packet to output 1. Transfer A (in1→out1). B stays at input 2. - t=2: Must transfer both B (in2→out1) and C (in2→out2). But both are at input 2 – a partial permutation allows at most one packet from each input port. Only one of B or C can cross the fabric. - If B crosses: B departs out1@t=2 ✓, but C delayed to t=3 ≠ t=2. ✗ - If C crosses: C departs out2@t=2 ✓, but B delayed to t=3 ≠ t=2. ✗ - Infeasible. ✗

### 4.2 – Internal Schedule for CIOQ with Speedup 2

With speedup 2, two partial permutations per time slot:

| Time Slot | Permutation 1 | Permutation 2 |
| --- | --- | --- |
| t = 1 | in1 → out1 (A) | in2 → out1 (B) |
| t = 2 | in2 → out2 (C) | — |

Result: A and B both reach output 1 by end of t=1. A departs t=1, B departs t=2. C reaches output 2 at t=2, departs t=2. Matches OQ schedule. ✓

## Question 5: TS 16×16 Switch Scheduling

### Setup

- 4×4 crossbar running at 4× port speed (4 sub-steps per time slot)

- Input lines: W(ports 1–4), X(ports 5–8), Y(ports 9–12), Z(ports 13–16)

- Output lines: A(ports 1–4), B(ports 5–8), C(ports 9–12), D(ports 13–16)

### Permutation

| Input | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Output | 7 | 10 | 5 | 11 | 1 | 16 | 15 | 14 | 2 | 12 | 4 | 13 | 9 | 3 | 6 | 8 |

### Traffic Matrix (connections per input-line/output-line pair)

Group connections by input line → output line: - W→A: none | W→B: 1→7, 3→5 | W→C: 2→10, 4→11 | W→D: none - X→A: 5→1 | X→B: none | X→C: none | X→D: 6→16, 7→15, 8→14 - Y→A: 9→2, 11→4 | Y→B: none | Y→C: 10→12 | Y→D: 12→13 - Z→A: 14→3 | Z→B: 15→6, 16→8 | Z→C: 13→9 | Z→D: none

Traffic matrix:

|  | A | B | C | D |
| --- | --- | --- | --- | --- |
| W | 0 | 2 | 2 | 0 |
| X | 1 | 0 | 0 | 3 |
| Y | 2 | 0 | 1 | 1 |
| Z | 1 | 2 | 1 | 0 |

Each row and column sums to 4. ✓

### Decomposition into 4 Permutation Matrices

| Sub-step | Crossbar Config | Connections Realized |
| --- | --- | --- |
| 1 | W→B, X→D, Y→A, Z→C | 3→5, 6→16, 9→2, 13→9 |
| 2 | W→C, X→D, Y→A, Z→B | 2→10, 7→15, 11→4, 15→6 |
| 3 | W→C, X→A, Y→D, Z→B | 4→11, 5→1, 12→13, 16→8 |
| 4 | W→B, X→D, Y→C, Z→A | 1→7, 8→14, 10→12, 14→3 |

Verification: All 16 port-to-port connections are covered exactly once. ✓

### Explanation of Computation

- Classify each of the 16 connections by (input line, output line) pair.

- Build the 4×4 traffic demand matrix.

- Decompose the matrix into 4 permutation matrices using iterative selection (each permutation = one valid crossbar configuration connecting each input line to exactly one output line).

- For each sub-step, assign specific port connections from the available pool for that (input line, output line) pair.

## Question 6: Traffic Shaping

### Flow Description

- 10 packets/second, each 1000 bits

- Packet transmission: 1 ms; gap between packets: 1 ms

- Burst duration: 19 ms (10 packets × 1ms + 9 gaps × 1ms)

- Idle period: 981 ms

- Pattern repeats every 1000 ms

### 6.1 – Flow Rate Function R(t)

R(t) is periodic with period T = 1 second = 1000 ms.

Within each period:

R(t)=1,000,000 bps=1 Mbpst∈[0,1)∪[2,3)∪[4,5)∪⋯∪[18,19) ms0otherwise

In other words: R(t) = 1 Mbps during ms intervals [2k, 2k+1) for k = 0, 1, …, 9 (mod 1000 ms), and R(t) = 0 elsewhere.

### 6.2 – Minimum Buffer Size σ (Drain Rate = 20 kbps)

The buffer fills during the burst and drains during idle. We track maximum buffer occupancy.

At the end of the k-th packet (t = 2k−1 ms, for k = 1, …, 10): - Bits input: 1000k - Bits drained: 20,000 bps × (2k−1)/1000 s = 20(2k−1) = 40k − 20 bits

Buffer occupancy = 1000k − (40k − 20) = 960k + 20

Maximum at k = 10 (t = 19 ms):

σmin=960×10+20=9,620 bits

Verification: After the burst, buffer drains at 20 kbps. Time to empty: 9620/20000 = 481 ms < 981 ms idle period. Buffer empties before next burst. ✓

### 6.3 – Dual Leaky Bucket Parameters

Bits are written to the buffer in groups of 2 bytes (16 bits) at a time.

The dual leaky bucket constraint requires: for any time interval of length t, the cumulative bits sent ≤ min(σ₁ + ρ₁·t, σ₂ + ρ₂·t).

First bucket (average rate + burst tolerance): - ρ₁ = 10,000 bps (average rate = 10 packets × 1000 bits / 1 s) - σ₁ = max excess above average rate over any interval

At end of k-th packet (t = 2k−1 ms): Excess = bits_sent − ρ₁·t = 1000k − 10,000·(2k−1)/1000 = 1000k − 20k + 10 = 980k + 10

Maximum at k = 10: σ₁ = 9,810 bits

Second bucket (peak rate constraint): - ρ₂ = 1,000,000 bps (peak rate = 1 Mbps during transmission) - σ₂ = 16 bits (one 2-byte group — the minimum instantaneous burst unit)

Final parameters:

ρ1=10,000 bps, σ1=9,810 bits, ρ2=1,000,000 bps, σ2=16 bits